In our implementation, we conduct experiments on an ARM platform to simulate a realistic edge environment. To ensure reflect real-world constraints, we do not use GPU acceleration or MPS, and all computations are performed on CPU.

Since we evaluate a hybrid framework, we simulate both edge and cloud components.

In the edge simulation, we first read data from the dataset, then apply feature scaling and feature selection to remove redundant or less informative features. After that, we perform feature extraction using methods such as PCA, AutoEncoder, or ResNeXt, and transmit the processed features to the cloud.

On the cloud side, the received data is reshaped based on metadata and then split into training and testing datasets. The data is then fed into a classification model for training and evaluation.

Finally, the classification results can be used to generate alerts or trigger defence strategies.

During this process, we monitor and record CPU and memory usage at the edge during feature extraction, as well as network traffic. We also evaluate model performance using metrics such as accuracy and F1 score.

In terms of we use the Edge-IIoTset dataset, which is a recent dataset designed for edge-based IoT environments(publish in 2022). It contains realistic network traffic and includes labelled data for both normal and attack behaviours, making it suitable for evaluating intrusion detection systems. In addition, the data is provided at the flow level, which aligns well with our system design and reduces computational complexity.

For feature reduction, we investigate several approaches. First, we apply feature selection methods based on feature value ranges and their importance for prediction, in order to remove redundant or less informative features. Then, we use PCA as a traditional dimensionality reduction technique to project the data into a lower-dimensional space. Finally, we explore deep learning-based methods, including AutoEncoder and ResNeXt1D, to learn more complex feature representations.

The main objective is to reduce the number of features while preserving the most important information, so that we can improve efficiency without significantly reducing classification performance.

In this part, we compare two feature extraction methods: ResNeXt1D and a lightweight DNN.

For ResNeXt1D, we first reshape the input data from a tabular format into a 1D format. Then, the data is passed through a stem layer, which includes convolution, batch normalisation, ReLU, and max pooling. This step helps extract basic features and reduce the data size.

After that, the data goes through several ResNeXt blocks to learn more complex features. Finally, we use pooling, flattening, and a linear layer to generate a fixed-size embedding.

For the DNN method, the input data is directly passed through fully connected layers with ReLU activation. The model gradually reduces the feature size and produces a compact embedding.

Compared to ResNeXt1D, the DNN is simpler, more efficient, and more suitable for tabular data and edge environments.

In this section, we evaluate the detection performance using accuracy and F1 score across different feature dimensions.

After Feature selection remains 52 features and achieves accuracy and F1 score approximetly 95%

From the results, we can see that the DNN model consistently achieves the best performance, with accuracy and F1 score close to 99%. It is also very stable across different dimensions.

ResNeXt also performs well, but it does not show clear improvement compared to DNN, especially considering its higher computational cost.

For PCA and AutoEncoder, the performance is lower when the feature dimension is small, and gradually improves as the dimension increases. This means that some important information is lost during dimensionality reduction.

Overall, these results show that for low-dimensional flow-level data, a simple DNN model is sufficient and more effective than more complex models.

In this experiment, we evaluate the computational cost using CPU, since most IoT and edge devices do not have GPUs.

From the CPU usage, we can see that ResNeXt has good parallel capability due to grouped convolution. However, even with this parallelism, the inference time is still much higher compared to other methods.

For memory usage, the results are not completely stable because Python memory management can introduce fluctuations. However, overall, the memory increase during inference is relatively small, generally below 10 MB.

When we consider both CPU time and memory usage together, ResNeXt has significantly higher computational cost, making it less suitable for edge deployment.

In contrast, PCA, AutoEncoder, and DNN have much lower overhead and are more efficient for edge-based IDS.

In this section, we evaluate the total latency of different methods. The latency is divided into three parts: encoding, network transmission, and inference.

For AutoEncoder, DNN, and PCA, the total latency is relatively low, generally between 60 and 150 milliseconds. Among them, DNN and AutoEncoder show stable performance across different feature dimensions.

However, for ResNeXt, the latency is significantly higher, reaching over 23,000 milliseconds. This is mainly because the encoding stage is very expensive.

This result shows that although ResNeXt can extract complex features, its computational cost is too high for real-time applications.

In contrast, PCA, AutoEncoder, and DNN provide much lower latency and are more suitable for edge-based intrusion detection systems.

Conclusion :

- Traditional dimensionality reduction techniques, such as PCA and AutoEncoder, can reduce the feature dimension by about 35% without significant loss in accuracy.

- For low-dimensional flow data, lightweight DNNs are more efficient and better suited for edge deployment than CNN-based models.

- ResNeXt introduces high computational overhead and is not suitable for edge deployment, while PCA, AutoEncoder, and DNN are more efficient for edge-based IDS.

Future work:

- Extend more classifiers, such as LightGBM and XGBoost, to test whether our features work well with different models.

- Move beyond flow-level classification by incorporating temporal and multi-flow detection, to better reflect real-world DDoS detection scenarios.

- Improve the explainability of the model by using methods like SHAP or permutation importance to understand which features are important.